| Date: |  | Conducted By: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bldg: |  |  | Room: |  | Dept: |  |
| Room Supervisor (PI if applicable): |  |  |  |  |  |  |

Safety and Risk Management recommends that an official Internal Self-Inspection is conducted annually for each space, as well as prior to each scheduled Workplace Inspection conducted by Safety & Risk or other organization. This is not a substitute for continual supervision and improvement, but rather a snapshot of proficiencies and deficiencies. Not all code violations are listed here, but rather the most common.   Each Section (Black Banners) may be applicable to every workspace, please review them all.  If you have any questions regarding these items or other workplace requirements, please contact SRM at 994-7760.

Yes = In compliance with the item

No = Violates the code for the item

CTI = Corrected at the Time of Inspection

N/A = Not Applicable in your workspace

Comments = Any Comments you would like us to take into consideration. Each Code Description has a minimum of 1 reference where the infraction may be found.  Some line items may have multiple references that relate to various work environments.  The individual that typically provides assistance for each line item is listed in parenthesis with their contact information at the end of the Checklist, although the Training and Development Coordinator is a great primary contact at 994-7760.

| Item | Code Description [Reference] (Assistance) | Yes | No | CTI | N/A | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Fire/Life Safety |  |  |  |  |  |
|  | Life Safety |  |  |  |  |  |
| 1.0 | Employer has furnished employment and place of employment which are free from recognized hazards that are causing or are likely to cause death or serious physical harm to his/her employees. [29 USC 654 Sec. 5 Duties] (Occupational Health Manager) |  |  |  |  |  |
|  | Electrical |  |  |  |  |  |
| 1.1 | Electrical rooms are found to be free of storage and clutter, with no combustibles found in the area. [2012 IFC 315.3] (MSU Fire Marshal) |  |  |  |  |  |
| 1.2 | Electrical panels have 36" of clear space for emergency access. [2012 IFC 605.3] (MSU Fire Marshal) |  |  |  |  |  |
| 1.3 | Electrical cords are in good condition and do not extend under doorways where they are exposed to excessive wear. [2012 IFC 605.1, 2012 IFC 605.5] (MSU Fire Marshal) |  |  |  |  |  |
| 1.4 | Power strips and splitters are UL certified and are not overloaded or daisy-chained. [2012 IFC 605.4, NFPA 70] (MSU Fire Marshal) |  |  |  |  |  |
| 1.5 | Extension cords shall not be a substitute for permanent wiring. They shall be used only with portable appliances, and plugged into wall receptacles. [2012 IFC 605.5] (MSU Fire Marshal) |  |  |  |  |  |
| 1.6 | All electrical sockets near a liquid source or area where liquids are used are protected by GFCI. [2012 IFC 605.1, 29 CFR 1910.303] (MSU Fire Marshal) |  |  |  |  |  |
| 1.7 | Electrical and Junction boxes have cover plates secured in place. [2012 IFC 605.6] (MSU Fire Marshal) |  |  |  |  |  |
|  | Emergency Equipment and Egress |  |  |  |  |  |
| 1.8 | Emergency exits and corridors are not obstructed to allow for emergency egress and access. [2012 IFC 1003.6, 29 CFR 1910.37] (MSU Fire Marshal) |  |  |  |  |  |
| 1.9 | Walkways and Aisles are uncluttered and free from tripping hazards. [2012 IFC 1018.3, 2012 IFC 315, 29 CFR 1910.22, 29 CFR 1910.176(c)] (MSU Fire Marshal) |  |  |  |  |  |
| 1.10 | Fire extinguishers are wall mounted with their tops no higher than 5' off the ground. [2012 IFC 906.7, 2012 IFC 906.9] (MSU Fire Marshal) |  |  |  |  |  |
| 1.11 | Fire extinguishers have a current certification tag. [NFPA 10.7.3.4 and 29 CFR 1910.157] (MSU Fire Marshal) |  |  |  |  |  |
| 1.12 | Fire extinguisher access is not restricted. [2012 IFC 906.6] (MSU Fire Marshal) |  |  |  |  |  |
| 1.13 | 18" of clearance is maintained from fire sprinkler head deflectors. [2012 IFC 315.3, 29 CFR 1910.159] (MSU Fire Marshal) |  |  |  |  |  |
| 1.14 | 24" of clearance is maintained from ceiling in non-fire sprinkled areas. [2012 IFC 315.3] (MSU Fire Marshal) |  |  |  |  |  |
| 1.15 | Fire Sprinkler Heads show no sign of corrosion, foreign materials, paint or physical damage. [NFPA 25-5.2.1.1] (MSU Fire Marshal) |  |  |  |  |  |
| 1.16 | No items are hung from or wrapped around sprinkler water lines or sprinkler heads. [NFPA 5.2.2.2] (MSU Fire Marshal) |  |  |  |  |  |
| 1.17 | Emergency Flushing Equipment (eyewash and shower) is located in areas that are accessible within 10 seconds (55') of areas of chemical use/storage. [ANSI Z358.1, 29 CFR 1910.151(c)] (Property Loss Prevention Manager) |  |  |  |  |  |
| 1.18 | An unobstructed path to emergency showers and eyewash stations is maintained, including 6' of clear space in front of the station. [ANSI Z358.1, 29 CFR 1910.151] (Property Loss Prevention Manager) |  |  |  |  |  |
| 1.19 | Eyewash stations are activated weekly (recommended 30 seconds) until all debris is removed and water runs clear and test is recorded on hang tag. [ANSI Z358.1, 29 CFR 1910.151(c)]. SRM will provide record tags upon request. (Property Loss Prevention Manager) |  |  |  |  |  |
| 1.20 | Adequate First Aid Supplies are readily available. [29 CFR 1910.151(b)] (Property Loss Prevention Manager) |  |  |  |  |  |
| 1.21 | All system valves, including sprinkler control valves, are protected from physical damage and shall be accessible. [NFPA 25-13.2.3] (MSU Fire Marshal) |  |  |  |  |  |
|  | Signage |  |  |  |  |  |
| 1.22 | Confined Spaces are labeled as such. [29 CFR 1910.146]. SRM will provide appropriate signage upon request. (Property Loss Prevention Manger) |  |  |  |  |  |
| 1.23 | Signage notifying entrants of hazards within a room and emergency contact information is present and up-to-date. Complete a Hazard Warning Signage Request and Chemical Inventory (if appropriate) to receive University approved signage. [29 CFR 1910.1200, 29 CFR 1910.1450, 29 USC 654 Sec. 5 Duties] (Training and Development Coordinator) |  |  |  |  |  |
|  | Miscellaneous |  |  |  |  |  |
| 1.24 | Clothes dryer is on preventative maintenance schedule to prevent lint build-up. [2012 IFC 2203.2, NFPA 1 – 11.5.1.11.1] (Property Loss Prevention Manager) |  |  |  |  |  |
| 1.25 | Housekeeping – all places of employment, passageways, storerooms, and service rooms are clean and orderly and in a sanitary condition. [29 CFR 1910.22 and 2012 IFC 305.3] (Property Loss Prevention Manager) |  |  |  |  |  |
| 1.26 | Shelving is equipped with restraints (lips, wires, or other barriers) to prevent items stored from falling off and secured to prevent racks from falling. [2012 IFC 5704.3.3.5.2, 29 CFR 1910.106, 29 CFR 1910.36, 29 CFR 1910.37, 29 CFR 1910.1450 Appendix A] (Property Loss Prevention Manager) |  |  |  |  |  |
| 1.27 | Fire Resistance rating of construction is maintained: Holes in fire rated walls are filled with appropriately rated fire caulking or similar. [2012 IFC 703.1, 2012 IFC 702] (MSU Fire Marshal) |  |  |  |  |  |
| 2 | General Waste |  |  |  |  |  |
| 2.1 | Waste and broken glassware is disposed of properly, not overfilled. [29 CFR 1910.1030, 29 USC 654 Sec. 5 Duties] (Occupational Health Manager) |  |  |  |  |  |
| 2.2 | Electronics are disposed of through the E-Scrap Program, or other appropriate recycling pick-up, if device is applicable. [40 CFR 261] (E-Scrap Coordinator) |  |  |  |  |  |
| 2.3 | Batteries are disposed of through the Hazardous Waste Program, or other appropriate recycling program. [40 CFR 273] (E-Scrap Coordinator) |  |  |  |  |  |
| 2.4 | Pesticides are disposed of through the Hazardous Waste Program or through appropriate State collection site. [40 CFR 273] (Chemical Safety Officer) |  |  |  |  |  |
| 2.5 | Mercury-Containing Articles (Not including lamps, see next 2 items) are disposed of through the Hazardous Waste Program, or other appropriate collection program. [40 CFR 273] (Chemical Safety Officer) |  |  |  |  |  |
| 2.6 | Intact Lamps (i.e. fluorescent, neon, etc.) are disposed of through the Universal Waste Program or other appropriate collection program. [40 CFR 273] (E-Scrap Coordinator) |  |  |  |  |  |
| 2.7 | Broken or leaking lamps, bulbs, and batteries are disposed of through the Hazardous Waste Program. [40 CFR 273] (Chemical Safety Officer) |  |  |  |  |  |
| 2.8 | Refrigerant containing equipment (refrigerators, freezers, air conditioners, etc.) are disposed of through Facilities Services Work Control [40 CFR 82] (Facilities Services Work Control) |  |  |  |  |  |
| 2.9 | Cylinders and lecture bottles are not thrown away without being properly discharged and secured. [NFPA 58, CGA] (Occupational Health Manager). |  |  |  |  |  |
| 3 | Personal Protective Equipment (PPE)/Clothing |  |  |  |  |  |
| 3.1 | Appropriate attire and PPE for working conditions are donned. [29 CFR 1910.132, 29 CFR 1910.1450] (Occupational Health Manager) |  |  |  |  |  |
| 3.2 | Appropriate PPE use is consistently observed while in restricted areas or while working with hazardous materials. [29 CFR 1910.132] (Chemical Safety Officer) |  |  |  |  |  |
| 3.3 | PPE is available to all employees and visitors within working areas. [29 CFR 1910.132] (Occupational Health Manager) |  |  |  |  |  |
| 4 | Portable Compressed Gas Cylinders (Cylinders) |  |  |  |  |  |
| 4.1 | Cylinders are standing up, and secured with a chain with a minimum thickness of .25”, or similar restraint. [2012 IFC 5303.5.3, CGA] (Occupational Health Manager) |  |  |  |  |  |
| 4.2 | Cylinders not in use are not stored in the laboratory unit. [NFPA 45-11.1.6.4] (Occupational Health Manager) |  |  |  |  |  |
| 4.3 | Cylinders are capped when not in use and have a pressure regulator when in use. [NFPA 45-11.1.6, 2012 IFC 5303.6] (Occupational Health Manager) |  |  |  |  |  |
| 5 | Bloodborne Pathogens |  |  |  |  |  |
| 5.0 | Laboratories working with Biological Agents, see additional checklist from Biosafety Program. (Biosafety Officer) |  |  |  |  |  |
| 5.1 | An exposure control plan is in place and all personnel have been trained on its contents. [29 CFR 1910.1030] (Biosafety Officer) |  |  |  |  |  |
| 5.2 | Biological waste is stored in appropriate Bio-waste hard-sided/plastic containers lined with red biohazard bags. [29 CFR 1910.1030] (Occupational Health Manager) |  |  |  |  |  |
| 5.3 | Used syringes and disposable needles are placed safely in appropriate sharps containers for disposal. [29 CFR 1910.1030] (Occupational Health Manager) |  |  |  |  |  |
| 5.4 | Containers used to store or transport human blood or OPIM are labeled as such. [29 CFR 1910.1030] (Occupational Health Manager) |  |  |  |  |  |
| 5.5 | Equipment exposed to human blood or OPIM is labeled appropriately. [29 CFR 1910.1030] (Occupational Health Manager) |  |  |  |  |  |
| 5.6 | Refrigerator or Freezer containing human blood, OPIM or Biohazardous materials labeled appropriately. [29 CFR 1910.1030] (Biosafety Officer) |  |  |  |  |  |
| 5.7 | Sharps containers or other biohazard red contains are not overfilled. [29 CFR 1910.1030(d)] (Occupational Health Manager) |  |  |  |  |  |
| 6 | Chemical Safety |  |  |  |  |  |
|  | General Chemical Safety |  |  |  |  |  |
| 6.1 | Current SDS/MSDS sheets for all chemicals and hazardous substances are accessible to all affected employees. [29 CFR 1910.1200] (Occupational Health Manager) |  |  |  |  |  |
| 6.2 | All chemicals are properly labeled with a product identifier and words, pictures, symbols, or combination thereof, which provide at least general information regarding the hazards of the chemicals, and which, in conjunction with the other information available to employees, will provide employees with the specific information regarding the physical and health hazards of the hazardous chemical. [29 CFR 1910.1200, NFPA 45-9.2.3.4] (Chemical Safety Officer) |  |  |  |  |  |
| 6.3 | Chemicals are dated when opened, or received, as appropriate (Peroxide forming agents in particular – 6 month maximum retention). [29 CFR 1910.1200, NFPA 45-9.2.3.4] (Chemical Safety Officer) |  |  |  |  |  |
| 6.4 | Chemical storage containers are in good condition, not degraded, and with their lids on securely when not in immediate use. [29 CFR 1910.1200, 29 CFR 1910.1450] (Chemical Safety Officer) |  |  |  |  |  |
| 6.5 | Water reactive chemicals are not to be stored near a water source. [29 CFR 1910.1200] (Chemical Safety Officer) |  |  |  |  |  |
| 6.6 | Chemicals are stored according to compatibility. [29 CFR 1910.1450, NFPA 45-9.2.3.2] (Chemical Safety Officer) |  |  |  |  |  |
| 6.7 | Flammable chemicals are stored in flammable cabinets appropriately, and away from sources of heat and ignition. [29 CFR 1910.1450, NFPA 45-9.2.3.3] (Chemical Safety Officer) |  |  |  |  |  |
| 6.8 | Storage of chemicals in the fume hood is prohibited. [NFPA 45-9.2.3.7] (Chemical Safety Officer) |  |  |  |  |  |
| 6.9 | Secondary containment is provided for liquid chemicals as needed, and used for dry chemicals next to or below liquids. Contact CSO for containers. [29 CFR 1910.1450, 40 CFR 262] (Chemical Safety Officer) |  |  |  |  |  |
| 6.10 | Rated refrigerators or freezers shall be used for flammable liquid storage where required below ambient temperatures. [NFPA 45-12.2.2] (Chemical Safety Officer) |  |  |  |  |  |
| 6.11 | Chemical Inventories are current. [29 CFR 1910.1450, 29 CFR 1910.1200, NFPA 704] (Chemical Safety Officer) |  |  |  |  |  |
| 6.12 | Non-Pressurized Cryogenic containers are moved with approved hand carts, hand trucks or other mobile devices designed for the secure movement of the container. [2012 IFC 5505.5, 2013 NFPA 55-8.14.11.4] (Chemical Safety Officer) |  |  |  |  |  |
|  | Chemical and Hazardous Waste |  |  |  |  |  |
| 6.13 | Hazardous Waste containers shall be in good condition, labeled and have closed lids. Request containers through Hazardous Waste Pick-Up. [NFPA 45] (Chemical Safety Officer) |  |  |  |  |  |
| 6.14 | Hazardous Waste shall be stored and mixed only if they are determined to be compatible by a qualified person. [NFPA 45] (Chemical Safety Officer) |  |  |  |  |  |
| 6.15 | Hazardous Waste shall be stored in secondary containment and are not stored near a drain or sink. Request containers through Hazardous Waste Pick-Up. [29 CFR 1910.1200, NFPA 45] (Chemical Safety Officer) |  |  |  |  |  |
| 6.16 | Chemicals may not be disposed of by evaporation into a fume hood, a room, or outside open air. [40 CFR 260.10] (Chemical Safety Officer) |  |  |  |  |  |
| 6.17 | Chemicals may only be disposed of through drains if it is in quantities less than 100mls and diluted 100 times. [EPA RCRA] (Chemical Safety Officer) |  |  |  |  |  |
| 6.18 | Chemical and Hazardous waste shall be disposed of through the Hazardous Waste Program. [40 CFR 261, 40 CFR 262] (Chemical Safety Officer) |  |  |  |  |  |
| 6.19 | All Chemical Waste containers must be labeled with the initial date of accumulation, the words “HAZARDOUS WASTE,” the responsible party’s name, a list of full chemical names and percentages of chemicals in the container. [40 CFR 261, 40 CFR 262] (Chemical Safety Officer) |  |  |  |  |  |
| 7 | Industrial Safety |  |  |  |  |  |
| 7.1 | Ladders present are rated for use and constructed of appropriate materials. [29 CFR 1910.25, 29 CFR 1926.1053] (Occupational Health Manager) |  |  |  |  |  |
| 7.2 | Machines and power tools are guarded appropriate to prevent injury. [29 CFR 1910.212] (Occupational Health Manager) |  |  |  |  |  |
| 7.3 | Engineered and power assisted tools are available and used when necessary. [29 USC 654 Sec. 5 Duties] (Occupational Health Manager) |  |  |  |  |  |
| 7.4 | Respirators shall be in good condition and stored according to the OSHA regulations: Cleaned and disinfected; protected from damage, contamination, dust, sunlight, extreme temperatures, excessive moisture, and damaging chemicals. [29 CFR 1910.134] (Occupational Health Manager) |  |  |  |  |  |
| 8 | Training |  |  |  |  |  |
| 8.1 | Personnel working in laboratory settings have received appropriate Laboratory Safety Trainings. [29 CFR 1910.1450] (Training and Development Coordinator) |  |  |  |  |  |
| 8.2 | Personnel have received trainings specific to their lab environment and processes. [29 CFR 1910.1450] (Training and Development Coordinator) |  |  |  |  |  |
| 8.3 | Personnel working with or near Hazardous Chemicals have received Hazard Communication Training post February 1, 2013. [29 CFR 1910.1200] (Training and Development Coordinator) |  |  |  |  |  |
| 8.4 | Personnel that work with Hazardous Chemicals shall have received hazard training on the chemicals, including the SDS and protocols. [29 CFR 1910.1200, 29 CFR 1910.1450, 29 USC 654 Sec. 5 Duties] (Training and Development Coordinator) |  |  |  |  |  |
| 8.5 | Personnel that work around hazards necessitating PPE shall have received training on PPE selection, use, and inspection. [29 CFR 1910.132] (Training and Development Coordinator) |  |  |  |  |  |
| 8.6 | Personnel working with Radioactive Materials have received Radioactive Materials Safety or Nuclear Gauge Safety training. [10 CFR Part 20] (Radiation Safety Officer) |  |  |  |  |  |
| 8.7 | Personnel working with Asbestos containing materials have received an appropriate level of Asbestos training. [29 CFR 1910.1001, 29 CFR 1926.1101] (Training and Development Coordinator) |  |  |  |  |  |
| 8.8 | Personnel operating Aerial Devices or Forklifts have received the appropriate training. [29 CFR 1926.454, 29 CFR 1910.178] (Training and Development Coordinator) |  |  |  |  |  |
| 8.9 | Personnel necessitating respirator protection are under Medical Surveillance and have received Respirator Training for their device. [29 CFR 1910.134] (Occupational Health Manager) |  |  |  |  |  |
| 8.10 | Personnel shall have received annual training on Portable Fire Extinguishers in areas where they are located. [29 CFR 1910.157] (Training and Development Coordinator) |  |  |  |  |  |
| 8.11 | Personnel that work with or are exposed to human blood, bodily fluids, tissues, cell lines, etc. have completed an annual Bloodborne Pathogens Training. [29 CFR 1910.1030] (Training and Development Coordinator) |  |  |  |  |  |
| 9 | Ergonomics |  |  |  |  |  |
| 9.1 | Chairs on casters all have a minimum of 5 casters and are in good working order. (Occupational Health Manager) |  |  |  |  |  |
| 9.2 | Personnel have and utilize ergonomic tools to prevent musculoskeletal injuries. (Occupational Health Manager) |  |  |  |  |  |
| 9.3 | Personnel that perform repetitive motions shall have breaks throughout the day, or have other intermittent tasks to prevent injuries. (Occupational Health Manager) |  |  |  |  |  |
| 10 | Laboratory Specific |  |  |  |  |  |
| 10.0 | Please refer to the Laboratory Specific Self-Inspection Sheet for additional criteria. |  |  |  |  |  |

Additional Comments by occupants or inspector:

- Occupational Health Manager - Laurie Bachar x7384

- Training and Development Coordinator - Trevor Hildebrand x7760

- MSU Fire Marshal - Skip Hougland x7583

- Radiation Safety Officer – Office of Research Compliance x7317

- Chemical Safety Officer - Patrick Ryan x7803

- Property Loss Prevention - Dan Archer x7597

- E-Scrap and Universal Waste - Shay Halverson x5678

- Industrial Hygiene and Safety – Patrick Schenck x1675

- Facilities Services Work Control x2107

- Biosafety Officer – Kirk Lubick x6998

- Sanitarian – Toots Taszut x6455

Waste Pick-up: https://www.montana.edu/wwwsrm/forms/waste/index.html

E-Scrap Pick-up: https://www.montana.edu/wwwsrm/forms/escrapform.php

Tools and Training: http://www.montana.edu/wwwsrm/